WiFi Pineapple Mark VII

Operator Reference & Co-Pilot Support Doc

Hak5 MK7 | Firmware 2.x | docs.hak5.org

# 1. Hardware Overview

| Spec | Value |
| --- | --- |
| CPU | Quad Core ARM Network SoC |
| Memory | 1GB RAM |
| Storage | 4GB eMMC |
| Wireless | 2.4/5GHz 802.11 a/b/g/n/ac/ac wave2 |
| Radios | Three 2.4GHz radios (Standard) / Four Dual-Band radios (Enterprise) |
| Antennas | 8x RP-SMA high gain |
| Ports | 2x Gigabit Ethernet, USB 3.0 host, USB-C Ethernet |
| Power | Wall-plug 120/220V |
| LED | 4x Configurable RGB status LEDs |

## Radio Role Assignment

| Interface | Default Role |
| --- | --- |
| wlan0 | Management AP (connects your computer to the Pineapple) |
| wlan1 | Recon / Deauth (scanning interface) |
| wlan2 | PineAP Open/Evil WPA AP |
| wlan3 | Available for MK7AC USB adapter (2.4+5GHz) |

## Connection Methods

| Method | Details |
| --- | --- |
| USB-C Ethernet | Plug into computer, ASIX AX88772C driver, DHCP 172.16.42.x |
| RJ45 LAN | Direct Ethernet to computer |
| WiFi | Connect to Pineapple_XXXX open network (XXXX = last 4 of MAC) |
| Web UI | http://172.16.42.1:1471 |
| SSH | ssh root@172.16.42.1 (port 22) |

# 2. Traffic Light Control System

The co-pilot operates on a three-level control system. Think of every action as a road — green means go, yellow means announce first, red means full stop.

GREEN LIGHT — Proceed autonomously without asking

- Navigate between any UI pages

- Read status, logs, scan results, client lists

- Configure PineAP mode and logging settings

- Start and stop recon scans

- Manage SSID impersonation pool

- Install and launch modules

- Adjust LED, hostname, cosmetic settings

- View handshakes and campaign results

- Toggle campaigns on/off

YELLOW LIGHT — Announce before doing, proceed unless Dallas says stop

- Enabling Evil WPA or Evil Enterprise access points

- Changing network/SSID settings that affect connectivity

- Modifying client or SSID filter rules

- Enabling PineAP Active mode (broadcasts to nearby devices)

- Changing management network configuration

- Firmware updates

- Connecting/disconnecting upstream networks

- SSH access

RED LIGHT — Full stop. Explicit GO required before proceeding.

- Factory reset

- Any action that would make the UI unreachable

- Deauth attacks against specific named targets

- Any action outside the authorized engagement scope

- Anything Dallas has not explicitly tasked

# 3. Web UI Navigation Map

All pages accessible from: http://172.16.42.1:1471

| Section | URL Hash | What It Does |
| --- | --- | --- |
| Dashboard | #/dashboard | System health, connected clients, campaigns, wireless landscape |
| Campaigns | #/campaigns | Automated pentest workflows chaining recon, PineAP, reporting |
| PineAP | #/pineap | Core rogue AP engine — all attack modes, filters, client tracking |
| Recon | #/recon | WiFi scanning, AP/client discovery, handshake auto-capture, deauth |
| Handshakes | #/handshakes | Captured WPA handshakes — download pcap or hashcat format |
| Modules | #/modules | Install and manage community modules |
| Settings | #/settings | Device config: networking, WiFi, LED, hostname, firmware, diagnostics |
| Cloud C2 | #/c2 | Provision remote management via Hak5 Cloud C2 |

## PineAP Tab Reference

| Tab | Function |
| --- | --- |
| Settings | Mode (Passive/Active/Advanced), event logging, SSID pool capture toggle |
| Open SSID | Basic open AP or impersonate-all mode |
| Evil WPA | WPA/WPA2 rogue AP — known PSK or handshake harvest mode |
| Evil Enterprise | EAP rogue AP — MSCHAPv2 hash capture or GTC plaintext capture |
| Impersonation | SSID pool management — manual add or auto-collect from probes/recon |
| Clients | Connected and previous client history with kick controls |
| Filtering | Client filter and SSID filter — Allow or Deny modes |
| Access Points | EvilWPA AP configuration and behavior settings |

# 4. PineAP Operation Modes

## Passive Mode

Collects nearby AP information, adds to SSID pool. Accepts connections to Open/WPA/Enterprise SSIDs if enabled. Does NOT broadcast or advertise other SSIDs. Lowest detection risk.

## Active Mode

Collects nearby APs AND actively broadcasts all SSIDs from the impersonation pool. Responds to all client probe requests allowed by filter rules. Maximum client capture. Higher detection risk.

## Advanced Mode

All PineAP features individually configurable. Mix and match exactly what you need for the engagement.

## Evil Enterprise Authentication Methods

| Method | Client Behavior | What Is Captured |
| --- | --- | --- |
| Any (recommended) | Allows any auth method | GTC gets plaintext; MSCHAPv2 gets challenge hash |
| MSCHAPv2 | Client receives error but connects | Challenge hash only — crack offline with hashcat |
| GTC | Client connects fully | Full plaintext username and password |

# 5. REST API Quick Reference

Base URL: http://172.16.42.1:1471/api/

Auth: Token-based. Obtain token via POST /api/login, include as Authorization header.

| Endpoint | Description |
| --- | --- |
| POST /api/login | Authenticate — returns session token |
| GET /api/dashboard | System stats (CPU, RAM, disk, clients) |
| GET /api/pineap/settings | Current PineAP configuration |
| POST /api/pineap/settings | Update PineAP configuration |
| POST /api/recon/start | Start a recon scan |
| GET /api/recon/results | Get latest scan results |
| GET /api/handshakes | List all captured handshakes |
| GET /api/clients | Currently connected clients |
| POST /api/campaigns | Create a new campaign |
| GET /api/notifications | System notifications |
| GET /api/modules | Installed modules list |

Example: Start Recon via curl (SSH)

TOKEN=$(curl -s -X POST http://172.16.42.1:1471/api/login \

-d '{"username":"root","password":"YOUR_PASS"}' | python3 -c \

"import sys,json; print(json.load(sys.stdin)['token'])")

curl -H "Authorization: $TOKEN" http://172.16.42.1:1471/api/recon/results

# 6. SSH Filesystem Reference

ssh root@172.16.42.1

| Path | Contents |
| --- | --- |
| /pineapple/modules/ | Installed modules |
| /etc/config/ | OpenWRT config files (network, wireless, pineap) |
| /var/log/ | System logs |
| /tmp/ | Temp files, runtime data |
| /root/ | Home directory |
| /sd/ | SD card mount (if present) |

## Useful Commands

| Command | Purpose |
| --- | --- |
| logread | View system log |
| logread -f | Follow system log live |
| ifconfig | Network interfaces |
| iwconfig | Wireless interfaces |
| iwlist wlan1 scan | Scan for nearby networks |
| opkg list-installed | Installed packages |
| pineapd status | PineAP daemon status |
| df -h | Disk usage |
| free -m | Memory usage |
| top | Process monitor |
| reboot | Reboot device (RED light) |

# 7. Notable Community Modules

| Module | What It Does |
| --- | --- |
| nmap | Network scanning and host discovery |
| tcpdump | Packet capture on any interface |
| Responder | LLMNR/NBT-NS/MDNS poisoning and credential capture |
| SSLsplit | Transparent SSL/TLS interception |
| DNS Spoof | Redirect DNS queries to attacker-controlled IP |
| Evil Portal | Captive portal for credential phishing |
| Deauth | Targeted 802.11 deauthentication |
| AutoSSH | Persistent reverse SSH tunnel |
| URLsnarf | Log HTTP URLs from network traffic |
| Kismet | Advanced wireless network detector |

Modules are installed from: PineAP > Modules > Get Modules

Installed to: /pineapple/modules/

Module dev: Angular frontend + Python 3 backend

# 8. Troubleshooting Quick Reference

| Problem | Fix |
| --- | --- |
| Can't reach 172.16.42.1:1471 | Check USB-C driver (ASIX AX88772C), confirm IP in 172.16.42.x range |
| UI unresponsive after config change | SSH in: logread, check pineapd status, reboot if needed |
| Clients not connecting to PineAP | Check filter rules — SSID must be in Allow list or not in Deny list |
| Recon shows no results | Confirm wlan1 not in use by another mode, check interface with iwconfig |
| Evil Enterprise cert generation slow | Normal — can take up to 5 minutes while gathering entropy |
| Module not appearing | Check /pineapple/modules/, relaunch module from UI, check logread |
| Internet not available on Pineapple | Settings > Networking > Client Mode — connect to upstream AP |
| Handshakes not capturing | Confirm Evil WPA is enabled AND SSID is in filter Allow list |
| Factory reset needed | Hold reset button per on-screen instructions (RED light — confirm first) |

# 9. Co-Pilot Communication Reference

These labels appear in every co-pilot message so you always know what mode it is operating in.

| Label | Meaning |
| --- | --- |
| [NAV] | Navigating or clicking in the UI |
| [READ] | Reading and interpreting current state |
| [CONFIG] | Changing a setting |
| [YELLOW — ANNOUNCING] | About to do a yellow light action — 3 seconds to say stop |
| [RED — REQUESTING APPROVAL] | Stopped, waiting for explicit GO |
| [HANDOFF] | Dallas needs to perform an action |
| [SSH] | Switching to command line |
| [PAGE REPORT] | Status update after navigation |
| [CONTEXT REFRESH] | Full session state summary |
| [SESSION CLOSE] | End of session summary |

# 10. Official Documentation Links

- Main docs: https://docs.hak5.org/wifi-pineapple/

- Developer docs: https://hak5.github.io/mk7-docs/

- REST API: https://hak5.github.io/mk7-docs/docs/rest/rest/

- Module development: https://hak5.github.io/mk7-docs/docs/modules/modules/

- Community forums: https://forums.hak5.org/

- GitHub modules repo: https://github.com/hak5/mk7-modules

- Cloud C2: https://c2.hak5.org

WiFi Pineapple Mark VII Operator Reference | Royal Tree Services Security Lab